The 1.8-inch TFT LCD module powered by the ST7735R driver IC is a compact color display frequently used in microcontrollers like Arduino, ESP32, and Raspberry Pi. [1, 2]

Key Technical Specifications

- Driver IC: Sitronix ST7735R (single-chip controller for 128x160 dot-matrix TFTs)

- Display Size: 1.8 inches diagonal (Active/Visible area: ~28.03 mm x 35.04 mm)

- Resolution: 128 x 160 pixels (RGB)

- Color Depth: 18-bit color (262,144 shades / RGB 656 format)

- Interface: 4-wire or 5-wire Serial Peripheral Interface (SPI)

- Operating Voltage (Logic / VCC): 3.3V to 5V logic compatible (typically powered via 3.3V or 5V VCC with onboard LDO regulator)

- Backlight: Dual-white LED backlight (PWM dimmable, ~2.9V to 3.3V forward voltage)

- Operating Temperature: -20°C to +70°C [1, 2, 3, 4, 5, 6]

Standard Pinout Definition

| Pin No. | Symbol | Description |
| --- | --- | --- |
| 1 | VCC / VDD | Power supply (3.3V to 5V depending on module breakout) |
| 2 | GND | Ground |
| 3 | CS | Chip Select pin (Active low) |
| 4 | RESET / RST | Reset pin (Active low) |
| 5 | A0 / DC | Data / Command selection pin |
| 6 | SDA / MOSI | SPI Serial Data Input |
| 7 | SCK / SCLK | SPI Serial Clock Input |
| 8 | LED / BL | Backlight control (Connect to 3.3V/5V or PWM) |

Software & Libraries

- Commonly used with the Adafruit ST7735 and Adafruit GFX Library in Arduino IDE.

- Initialization command frequently depends on the tab color on the physical screen protection film (e.g., tft.initR(INITR_BLACKTAB) or INITR_REDTAB). [1, 2]

You can review complete board schematics and integration files via resources like the Adafruit 1.8" TFT Learning Guide or the DigiKey Adafruit 358 Product Page. [1, 2]