1.8" TFT Display Breakout and Shield

128x160 pixels in 18 bit color with a microSD

by lady ada and 2 other contributors

published August 29, 2012, last edited January 22, 2025
last major update January 24, 2023

posted in Arduino Compatibles LCDs & Displays/ Graphic TFTs Breakout Boards/ LCDs, LEDs, & Displays

Save Link Note Download

- Overview

- 1.8" TFT Breakout

- 1.8" TFT Shield V2

- Original V1 Shield

- Graphics Library

- Troubleshooting

- Downloads

- Multiple pages

- Feedback? Corrections?

- Text View

Guide Products view all

Featured

Adafruit Feather M4 Express - Featuring ATSAMD51

$22.95

Featured

Adafruit Metro M4 feat. Microchip ATSAMD51

$27.50

Featured

Adafruit Grand Central M4 Express featuring the SAMD51

$39.95

Featured

Adafruit Metro M4 Express AirLift (WiFi) - Lite

$34.95

- View product details

44

Beginner

Product guide

Overview

This tutorial is for our 1.8" diagonal TFT display. It comes packaged as a breakout or as an Arduino shield. Both styles have a microSD interface for storing files and images. These are both great ways to add a small, colorful and bright display to any project. Since the display uses 4-wire SPI to communicate and has its own pixel-addressable frame buffer, it requires little memory and only a few pins. This makes it ideal for use with small microcontrollers.

This display breakout comes with an EYESPI connector! This 18-pin 0.5mm pitch FPC connector has a flip-top connector for using a flex cable to hook up your display. It enables you to avoid soldering and get your display up off of the breadboard! Consider it a sort of "STEMMA QT for displays" - a way to quickly connect and extend display wiring that uses a lot of SPI pins. It also allows for communicating with displays over longer distances. The EYESPI flex cables are available in multiple lengths to suit any project. This is especially useful for projects where you want your display mounted separate from your microcontroller.

The shield version plugs directly into an Arduino with no wiring required. The breakout version can be used with every kind of microcontroller.

The 1.8" display has 128x160 color pixels. Unlike the low cost "Nokia 6110" and similar LCD displays, which are CSTN type and thus have poor color and slow refresh, this display is a true TFT! The TFT driver (ST7735R) can display full 18-bit color (262,144 shades!). And the LCD will always come with the same driver chip so there's no worries that your code will not work from one to the other. 

Both boards have the TFT soldered on (it uses a delicate flex-circuit connector) as well as a ultra-low-dropout 3.3V regulator and a 3/5V level shifter so you can use it with 3.3V or 5V power and logic. These also include a microSD card holder so you can easily load full color bitmaps from a FAT16/FAT32 formatted microSD card. And on the Shield version, we've added a nifty 5-way joystick navigation switch!

You can pick up one of these displays in the Adafruit shop!
1.8" 18-bit color TFT breakout
1.8" 18-bit Color TFT Shield

Page last edited March 08, 2024

Text editor powered by tinymce.

1.8" TFT Breakout

This lovely little display breakout is the best way to add a small, colorful and bright display to any project. Since the display uses 4-wire SPI to communicate and has its own pixel-addressable frame buffer, it can be used with every kind of microcontroller. Even a very small one with low memory and few pins available!

The 1.8" display has 128x160 color pixels. Unlike the low cost "Nokia 6110" and similar LCD displays, which are CSTN type and thus have poor color and slow refresh, this display is a true TFT! The TFT driver (ST7735R) can display full 18-bit color (262,144 shades!). And the LCD will always come with the same driver chip so there's no worries that your code will not work from one to the other.

The breakout has the TFT display soldered on (it uses a delicate flex-circuit connector) as well as a ultra-low-dropout 3.3V regulator and a 3/5V level shifter so you can use it with 3.3V or 5V power and logic. We also had a little space so we placed a microSD card holder so you can easily load full color bitmaps from a FAT16/FAT32 formatted microSD card. The microSD card is not included, but you can pick one up here.

This display breakout also features an 18-pin "EYESPI" standard FPC connector with flip-top connector. You can use an 18-pin 0.5mm pitch FPC cable to connect to all the GPIO pins for when you want to skip the soldering.

Page last edited March 08, 2024

Text editor powered by tinymce.

Pinouts

EYESPI

This display comes with an EYESPI connector, which is an 18pin 0.5mm pitch connector that allows you to use a flex cable to connect your display to your microcontroller. For more details, visit the EYESPI page.

This color display uses SPI to receive image data. That means you need at least 4 pins - clock, data in, tft cs and d/c. If you'd like to have SD card usage too, add another 2 pins - data out and card cs. However, there's a couple other pins you may want to use, lets go thru them all!

- Lite - this is the PWM input for the backlight control. Connect to 3-5VDC to turn on the backlight. Connect to ground to turn it off. Or, you can PWM at any frequency.

- MISO - this is the SPI Microcontroller In Serial Out pin, its used for the SD card. It isn't used for the TFT display which is write-only

- SCLK - this is the SPI clock input pin

- MOSI - this is the SPI Microcontroller Out Serial In pin, it is used to send data from the microcontroller to the SD card and/or TFT

- TFT_CS - this is the TFT SPI chip select pin

- Card CS - this is the SD card chip select, used if you want to read from the SD card.

- D/C - this is the TFT SPI data or command selector pin

- RST - this is the TFT reset pin. Connect to ground to reset the TFT! Its best to have this pin controlled by the library so the display is reset cleanly, but you can also connect it to the Arduino Reset pin, which works for most cases.

- Vcc - this is the power pin, connect to 3-5VDC - it has reverse polarity protection but try to wire it right!

- GND - this is the power and signal ground pin

For the level shifter we use the CD74HC4050 which has a typical propagation delay of ~10ns

Page last edited March 08, 2024

Text editor powered by tinymce.

EYESPI

This display now comes with an EYESPI connector. This connector allows you to connect your display without soldering. There are EYESPI cables available in multiple lengths, which means you can find one to fit any project. This is especially useful if your project requires the display to be freestanding, and not tied directly into a breadboard. Inspired by the popularity of STEMMA QT, it provides plug-n-play for displays!

The EYESPI Connector and Cables

The EYESPI connector is an 18 pin 0.5mm pitch FPC connector with a flip-top tab for locking in the associated flex cable. It is designed to allow you to connect a display, without needing to solder headers or wires to the display.

The EYESPI connector location on this display is indicated below.

The EYESPI cables are 18 pin 0.5mm pitch flex cables. They are ~9.6mm wide, and designed to fit perfectly into the EYESPI connector. Adafruit currently offers EYESPI cables in three different lengths: 50mm, 100mm, and 200mm.

Text emphasized with a yellow exclamation: The EYESPI connector is designed to work with 18-pin 0.5mm pitch flex cables. Other flex cables, such as Raspberry Pi camera flex cables, will not work!

Wiring Your EYESPI Display

Wiring your EYESPI display to a microcontroller via the EYESPI connector requires the EYESPI breakout board and an EYESPI cable.

Your browser does not support the video tag.

Featured Adafruit EYESPI Breakout Board - 18 Pin FPC Connector

Our most recent display breakouts have come with a new...

$1.95 each

In Stock

Featured EYESPI Cable - 18 Pin 100mm long Flex PCB (FPC) A-B type

Connect this to that when a 18-pin FPC connector is needed. This 25 cm long cable is made of a flexible PCB. It's...

$0.95 each

In Stock

The following example shows how to connect the 1.8" TFT Display Breakout to a Feather RP2040 using the EYESPI breakout board.

Connect the following Feather pins to the associated EYESPI breakout pins:

- breakout Vin to Feather 3.3V (red wire)

- breakout Lite to Feather 3.3V (yellow wire)

- breakout Gnd to Feather GND (black wire)

- breakout SCK to Feather SCK (grey wire)

- breakout MISO to Feather MI (green wire)

- breakout MOSI to Feather MO (purple wire)

- breakout TCS to Feather D5 (blue wire)

- breakout DC to Feather D6 (orange wire)

- breakout RST to Feather D9 (cyan wire)

- breakout SDCS to Feather D10 (pink wire)

Finally, connect your display EYESPI connector to the breakout EYESPI connector using an EYESPI cable. For details on using the EYESPI connector properly, visit Plugging in an EYESPI Cable.

EYESPI Pins

Though there are 18 pins available on the EYESPI connector, many displays do not use all available pins. This display requires the following pins:

- Vin - This is the power pin. To power the board (and thus your display), connect to the same power as the logic level of your microcontroller, e.g. for a 3V micro like a Feather, use 3V, and for a 5V micro like an Arduino, use 5V.

- Lite - This is the PWM input for the backlight control. It is by default pulled high (backlight on), however, you can PWM at any frequency or pull down to turn the backlight off.

- Gnd - This is common ground for power and logic.

- MISO - This is the SPI MISO (Microcontroller In / Serial Out) pin. It's used for the SD card. It isn't used for the display because it's write-only. It is 3.3V logic out (but can be read by 5V logic).

- MOSI - This is the SPI MOSI (Microcontroller Out / Serial In) pin. It is used to send data from the microcontroller to the SD card and/or display.

- SCK - This is the SPI clock input pin.

- TCS - This is the TFT SPI chip select pin.

- RST - This is the display reset pin. Connecting to ground resets the display! It's best to have this pin controlled by the library so the display is reset cleanly, but you can also connect it to the microcontroller's Reset pin, which works for most cases. Often, there is an automatic-reset chip on the display which will reset it on power-up, making this connection unnecessary in that case.

- DC - This is the display SPI data/command selector pin.

- SDCS - This is the SD card chip select pin. This pin is required for communicating with the SD card holder onboard the connected display.

Page last edited March 08, 2024

Text editor powered by tinymce.

Plugging in an EYESPI Cable

You can connect an EYESPI compatible display to the EYESPI breakout board using an EYESPI cable. An EYESPI cable is an 18 pin flexible PCB (FPC). The FPC can only be connected properly in one orientation, so be sure to follow the steps below to ensure that your display and breakout are plugged in properly.

Your browser does not support the video tag.

Each EYESPI cable has blue stripes on either end. On the other side of the cable, underneath the blue stripe, are the connector pins that make contact with the FPC connector pins on the display or breakout.

Your browser does not support the video tag.

To begin inserting an EYESPI cable to an FPC connector, gently lift the FPC connector black latch up.

Your browser does not support the video tag.

Then, insert the EYESPI cable into the open FPC connector by sliding the cable into the connector. You want to see the blue stripe facing up towards you. This inserts the cable pins into the FPC connector.

Your browser does not support the video tag.

To secure the cable, lower the FPC connector latch onto the EYESPI cable.

Repeat this process for the FPC connector on your display. Again, ensure that the blue stripe on either end of the cable is facing up.

Page last edited March 08, 2024

Text editor powered by tinymce.

Breakout Assembly

Prepare the header strip:

Cut the strip to length if necessary. It will be easier to solder if you insert it into a breadboard - long pins down

Add the breakout board:

Place the breakout board over the pins so that the short pins poke through the breakout pads

And Solder!

Be sure to solder all pins for reliable electrical contact.

(For tips on soldering, be sure to check out our Guide to Excellent Soldering).

You're done! Check your solder joints visually and continue onto the next steps

Page last edited March 08, 2024

Text editor powered by tinymce.

Breakout Wiring & Test

There are two ways to wire up these displays - one is a more flexible method (you can use any pins on the Arduino) and the other is much faster (4-8x faster, but you are required to use the hardware SPI pins) We will begin by showing how to use the faster method, you can always change the pins later for flexible 'software SPI'

Wiring up the display in SPI mode is pretty easy as there's not that many pins! We'll be using hardware SPI, but you can also use software SPI (any pins) later. Start by connecting the power pins

- 3-5V Vin connects to the Arduino 5V pin - red wires

- GND connects to Arduino ground - black wires

- CLK connects to SPI clock. On Arduino Uno/Duemilanove/328-based, thats Digital 13. On Mega's, its Digital 52 and on Leonardo/Due its ICSP-3 (See SPI Connections for more details) - this is the orange wire

- MOSI connects to SPI MOSI. On Arduino Uno/Duemilanove/328-based, thats Digital 11. On Mega's, its Digital 51 and on Leonardo/Due its ICSP-4 (See SPI Connections for more details) - this is the white wire

- CS connects to our SPI Chip Select pin. We'll be using Digital 10 but you can later change this to any pin - this is the yellow wire

- RST connects to our TFT reset pin. We'll be using Digital 9 but you can later change this pin too - this is the blue wire

- D/C connects to our SPI data/command select pin. We'll be using Digital 8 but you can later change this pin too - this is the green wire

Install Adafruit ST7735 TFT Library

We have example code ready to go for use with these TFTs. It's written for Arduino, which should be portable to any microcontroller by adapting the C++ source.

Three libraries need to be installed using the Arduino Library Manager…this is the preferred and modern way. From the Arduino “Sketch” menu, select “Include Library” then “Manage Libraries…”

Search for and install the Adafruit GFX library:

And the Adafruit ST7735 library:

If using an older version of the Arduino IDE (pre-1.8.10), also locate and install the Adafruit_BusIO library (newer versions do this automatically when using the Arduino Library Manager).

If this is all unfamiliar, we have a tutorial introducing Arduino library concepts and installation.

Restart the IDE!

After restarting the Arduino software, you should see a new example folder called Adafruit_ST7735 and inside, an example called graphicstest.

Now upload the sketch to your Arduino. You may need to press the Reset button to reset the arduino and TFT. You should see a collection of graphical tests draw out on the TFT.

Once uploaded, the Arduino should perform all the test display procedures! If you're not seeing anything - first check if you have the backlight on, if the backlight is not lit something is wrong with the power/backlight wiring. If the backlight is lit but you see nothing on the display make sure you're using our suggested wiring.

Changing Pins

Now that you have it working, there's a few things you can do to change around the pins.

If you're using Hardware SPI, the CLOCK and MOSI pins are 'fixed' and cant be changed. But you can change to software SPI, which is a bit slower, and that lets you pick any pins you like. Find these lines:

Download File

Copy Code

// Option 1 (recommended): must use the hardware SPI pins

// (for UNO thats sclk = 13 and sid = 11) and pin 10 must be

// an output. This is much faster - also required if you want

// to use the microSD card (see the image drawing example)

Adafruit_ST7735 tft = Adafruit_ST7735(TFT_CS,  TFT_DC, TFT_RST);

// Option 2: use any pins but a little slower!

#define TFT_SCLK 13   // set these to be whatever pins you like!

#define TFT_MOSI 11   // set these to be whatever pins you like!

//Adafruit_ST7735 tft = Adafruit_ST7735(TFT_CS, TFT_DC, TFT_MOSI, TFT_SCLK, TFT_RST);

Comment out option 1, and uncomment option 2. Then you can change the TFT_ pins to whatever pins you'd like!

You can also save a pin by setting

#define TFT_RST    9

to

#define TFT_RST   -1

and connecting the RST line to the Arduino Reset pin. That way the Arduino will auto-reset the TFT as well.

Page last edited March 08, 2024

Text editor powered by tinymce.

Displaying Bitmaps

In this example, we'll show how to display a 128x160 pixel full color bitmap from a microSD card.

We have an example sketch in the library showing how to display full color bitmap images stored on an SD card. You'll need a microSD card such as this one.

It's really easy to draw bitmaps. We have a library for it, Adafruit_ImageReader, which can be installed through the Arduino Library Manager (Sketch→Include Library→Manage Libraries…). Enter “imageread” in the search field and the library is easy to spot:

You'll also need an image. We suggest starting with this bitmap of a parrot.

If you want to later use your own image, use an image editing tool and crop your image to no larger than 160 pixels high and 128 pixels wide. Save it as a 24-bit color BMP file - it must be 24-bit color format to work, even if it was originally a 16-bit color image - because of the way BMPs are stored and displayed!

Copy the parrot.bmp to the microSD card and insert it into the micro SD card holder on your shield or breakout board.

Breakout Wiring

Shield users can skip directly to the "Example Sketch" section.

Wire up the TFT as described on the wiring & test page and add the two wires for talking to the SD card. Connect CARD_CS (the unconnected pin in the middle) to digital pin 4 (you can change this later to any pin you want). Connect MISO (second from the right) to the Arduino's hardware SPI MISO pin. For Classic arduinos, this is pin 12. For Mega's this is pin 50. You can't change the MISO pin, it's fixed in the chip hardware.

Example Sketch

If you have the breakout, open the File→examples→Adafruit ImageReader Library→BreakoutST7735 - 160x128 example.

If you have the shield, open the File→examples→Adafruit ImageReader Library→ShieldST7735 example.

Now upload the example sketch to the Arduino. It should display the parrot image. If you have any problems, check the serial console for any messages such as not being able to initialize the microSD card or not finding the image.

Page last edited March 08, 2024

Text editor powered by tinymce.

CircuitPython Displayio Quickstart

You will need a board capable of running CircuitPython such as the Metro M0 Express or the Metro M4 Express. You can also use boards such as the Feather M0 Express or the Feather M4 Express. We recommend either the Metro M4 or the Feather M4 Express because it's much faster and works better for driving a display. For this guide, we will be using a Feather M4 Express. The steps should be about the same for the Feather M0 Express or either of the Metros. If you haven't already, be sure to check out our Feather M4 Express guide.

Featured Adafruit Feather M4 Express - Featuring ATSAMD51

It's what you've been waiting for, the Feather M4 Express featuring ATSAMD51. This Feather is fast like a swift,...

$22.95 each

In Stock

Preparing the Breakout

Before using the TFT Breakout, you will need to solder the headers or some wires to it. Be sure to check out the Adafruit Guide To Excellent Soldering. After that the breakout should be ready to go.

Wiring the Breakout to the Feather

- 3-5V VCC connects to the Feather 3V pin

- GND connects toFeather ground

- SCK connects to SPI clock. On the Feather that's SCK.

- MISO connects to SPI MISO. On the Feather that's MI

- MOSI connects to SPI MOSI. On the Feather that's MO

- TFT_CS connects to our SPI Chip Select pin. We'll be using Digital 5 but you can later change this to any pin

- D/C connects to our SPI data/command select pin. We'll be using Digital 6 but you can later change this pin too.

- RESET connects to our reset pin. We'll be using Digital 9 but you can later change this pin too.

- LITE connects to the Feather 3V pin. This is the only display that this pin is required to be connected or the backlight won't work.

Required CircuitPython Libraries

To use this display with displayio, there is only one required library.

First, make sure you are running the latest version of Adafruit CircuitPython for your board.

Next, you'll need to install the necessary libraries to use the hardware--carefully follow the steps to find and install these libraries from Adafruit's CircuitPython library bundle.  Our introduction guide has a great page on how to install the library bundle for both express and non-express boards.

Remember for non-express boards, you'll need to manually install the necessary libraries from the bundle:

- adafruit_st7735r

Before continuing make sure your board's lib folder or root filesystem has the adafruit_st7735r file copied over.

Code Example Additional Libraries

For the Code Example, you will need an additional library. We decided to make use of a library so the code didn't get overly complicated.

Go ahead and install this in the same manner as the driver library by copying the adafruit_display_text folder over to the lib folder on your CircuitPython device.

CircuitPython Code Example

Download Project Bundle

Copy Code

# SPDX-FileCopyrightText: 2021 ladyada for Adafruit Industries

# SPDX-License-Identifier: MIT

"""

This test will initialize the display using displayio and draw a solid green

background, a smaller purple rectangle, and some yellow text.

"""

import board

import displayio

import terminalio

from adafruit_display_text import label

from fourwire import FourWire

from adafruit_st7735r import ST7735R

# Release any resources currently in use for the displays

displayio.release_displays()

spi = board.SPI()

tft_cs = board.D5

tft_dc = board.D6

display_bus = FourWire(spi, command=tft_dc, chip_select=tft_cs, reset=board.D9)

display = ST7735R(display_bus, width=160, height=128, rotation=90, bgr=True)

# Make the display context

splash = displayio.Group()

display.root_group = splash

color_bitmap = displayio.Bitmap(160, 128, 1)

color_palette = displayio.Palette(1)

color_palette[0] = 0x00FF00  # Bright Green

bg_sprite = displayio.TileGrid(color_bitmap, pixel_shader=color_palette, x=0, y=0)

splash.append(bg_sprite)

# Draw a smaller inner rectangle

inner_bitmap = displayio.Bitmap(150, 118, 1)

inner_palette = displayio.Palette(1)

inner_palette[0] = 0xAA0088  # Purple

inner_sprite = displayio.TileGrid(inner_bitmap, pixel_shader=inner_palette, x=5, y=5)

splash.append(inner_sprite)

# Draw a label

text_group = displayio.Group(scale=2, x=11, y=64)

text = "Hello World!"

text_area = label.Label(terminalio.FONT, text=text, color=0xFFFF00)

text_group.append(text_area)  # Subgroup for text scaling

splash.append(text_group)

while True:

pass

Show less View on GitHub

Let's take a look at the sections of code one by one. We start by importing the board so that we can initialize SPI, displayio,terminalio for the font, a label, and the adafruit_st7735r driver.

Download File

Copy Code

import board

import displayio

import terminalio

from adafruit_display_text import label

from adafruit_st7735r import ST7735R<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Next we release any previously used displays. This is important because if the Feather is reset, the display pins are not automatically released and this makes them available for use again.

Copy Code

displayio.release_displays()<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Next, we set the SPI object to the board's SPI with the easy shortcut function board.SPI(). By using this function, it finds the SPI module and initializes using the default SPI parameters.

Copy Code

spi = board.SPI()

tft_cs = board.D5

tft_dc = board.D6<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

In the next line, we set the display bus to FourWire which makes use of the SPI bus.

Copy Code

display_bus = displayio.FourWire(spi, command=tft_dc, chip_select=tft_cs, reset=board.D9)<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Finally, we initialize the driver with a width of 160 and a height of 128. If we stopped at this point and ran the code, we would have a terminal that we could type at and have the screen update. Because we want to use the display horizontally and the default orientation is vertical, we rotate it 90 degrees. One other parameter that we provide is bgr=True and the reason for this is that the color ordering of certain displays is Blue, Green, Red rather than the usual Red, Green, Blue. It tell displayio the correct color ordering for this particular display.

Copy Code

display = ST7735R(display_bus, width=160, height=128, rotation=90, bgr=True)

<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Next we create a background splash image. We do this by creating a group that we can add elements to and adding that group to the display. In this example, we are limiting the maximum number of elements to 10, but this can be increased if you would like. The display will automatically handle updating the group.

Copy Code

splash = displayio.Group(max_size=10)

display.root_group = splash

Next we create a Bitmap which is like a canvas that we can draw on. In this case we are creating the Bitmap to be the same size as the screen, but only have one color. The Bitmaps can currently handle up to 256 different colors. We create a Palette with one color and set that color to 0x00FF00 which happens to be green. Colors are Hexadecimal values in the format of RRGGBB. Even though the Bitmaps can only handle 256 colors at a time, you get to define what those 256 different colors are.

Copy Code

color_bitmap = displayio.Bitmap(160, 128, 1)

color_palette = displayio.Palette(1)

color_palette[0] = 0x00FF00 # Bright Green<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

With all those pieces in place, we create a TileGrid by passing the bitmap and palette and draw it at (0, 0) which represents the display's upper left.

Copy Code

bg_sprite = displayio.TileGrid(color_bitmap,

pixel_shader=color_palette,

x=0, y=0)

splash.append(bg_sprite)<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Next we will create a smaller purple square. The easiest way to do this is the create a new bitmap that is a little smaller than the full screen with a single color and place it in a specific location. In this case, we will create a bitmap that is 5 pixels smaller on each side. The screen is 160x128, so we'll want to subtract 10 from each of those numbers.

We'll also want to place it at the position (5, 5) so that it ends up centered.

Download File

Copy Code

inner_bitmap = displayio.Bitmap(150, 118, 1)

inner_palette = displayio.Palette(1)

inner_palette[0] = 0xAA0088 # Purple

inner_sprite = displayio.TileGrid(inner_bitmap,

pixel_shader=inner_palette,

x=5, y=5)

splash.append(inner_sprite)<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Since we are adding this after the first square, it's automatically drawn on top. Here's what it looks like now.

Next let's add a label that says "Hello World!" on top of that. We're going to use the built-in Terminal Font and scale it up by a factor of two. To scale the label only, we will make use of a subgroup, which we will then add to the main group.

Labels are centered vertically, so we'll place it at 64 for the Y coordinate, and around 11 pixels make it appear to be centered horizontally, but if you want to change the text, change this to whatever looks good to you. Let's go with some yellow text, so we'll pass it a value of 0xFFFF00.

Download File

Copy Code

text_group = displayio.Group(max_size=10, scale=2, x=11, y=64)

text = "Hello World!"

text_area = label.Label(terminalio.FONT, text=text, color=0xFFFF00)

text_group.append(text_area) # Subgroup for text scaling

splash.append(text_group)<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Finally, we place an infinite loop at the end so that the graphics screen remains in place and isn't replaced by a terminal.

Copy Code

while True:

pass<div class="open_grepper_editor" title="Edit & Save To Grepper"></div><div class="open_grepper_editor" title="Edit & Save To Grepper"></div>

Where to go from here

Be sure to check out this excellent guide to CircuitPython Display Support Using displayio

Page last edited March 08, 2024

Text editor powered by tinymce.

Python Wiring and Setup

Wiring

It's easy to use display breakouts with Python and the Adafruit CircuitPython RGB Display module.  This module allows you to easily write Python code to control the display.

We'll cover how to wire the display to your Raspberry Pi. First assemble your display.

Since there's dozens of Linux computers/boards you can use we will show wiring for Raspberry Pi. For other platforms, please visit the guide for CircuitPython on Linux to see whether your platform is supported.

Connect the display as shown below to your Raspberry Pi.

Text emphasized with a blue exclamation: Note this is not a kernel driver that will let you have the console appear on the TFT. However, this is handy when you can't install an fbtft driver, and want to use the TFT purely from 'user Python' code!

Text emphasized with a yellow exclamation: You can only use this technique with Linux/computer devices that have hardware SPI support, and not all single board computers have an SPI device so check before continuing

ILI9341 and HX-8357-based Displays

2.2" Display

- CLK connects to SPI clock. On the Raspberry Pi, that's SCLK

- MOSI connects to SPI MOSI. On the Raspberry Pi, that's also MOSI

- CS connects to our SPI Chip Select pin. We'll be using CE0

- D/C connects to our SPI Chip Select pin. We'll be using GPIO 25, but this can be changed later.

- RST connects to our Reset pin. We'll be using GPIO 24 but this can be changed later as well.

- Vin connects to the Raspberry Pi's 3V pin

- GND connects to the Raspberry Pi's ground

2.4", 2.8", 3.2", and 3.5" Displays

These displays are set up to use the 8-bit data lines by default. We want to use them for SPI. To do that, you'll need to either solder bridge some pads on the back or connect the appropriate IM lines to 3.3V with jumper wires. Check the back of your display for the correct solder pads or IM lines to put it in SPI mode.

- Vin connects to the Raspberry Pi's 3V pin

- GND connects to the Raspberry Pi's ground

- CLK connects to SPI clock. On the Raspberry Pi, thats SLCK

- MOSI connects to SPI MOSI. On the Raspberry Pi, thats also MOSI

- CS connects to our SPI Chip Select pin. We'll be using CE0

- D/C connects to our SPI Chip Select pin. We'll be using GPIO 25, but this can be changed later.

- RST connects to our Reset pin. We'll be using GPIO 24 but this can be changed later as well.

Text emphasized with a yellow exclamation: These larger displays are set to use 8-bit data lines by default and may need to be modified to use SPI.

ST7789 and ST7735-based Displays

1.3", 1.54", and 2.0" IPS TFT Display

- Vin connects to the Raspberry Pi's 3V pin

- GND connects to the Raspberry Pi's ground

- CLK connects to SPI clock. On the Raspberry Pi, thats SLCK

- MOSI connects to SPI MOSI. On the Raspberry Pi, thats also MOSI

- CS connects to our SPI Chip Select pin. We'll be using CE0

- RST connects to our Reset pin. We'll be using GPIO 24 but this can be changed later.

- D/C connects to our SPI Chip Select pin. We'll be using GPIO 25, but this can be changed later as well.

0.96", 1.14", and 1.44" Displays

- Vin connects to the Raspberry Pi's 3V pin

- GND connects to the Raspberry Pi's ground

- CLK connects to SPI clock. On the Raspberry Pi, thats SLCK

- MOSI connects to SPI MOSI. On the Raspberry Pi, thats also MOSI

- CS connects to our SPI Chip Select pin. We'll be using CE0

- RST connects to our Reset pin. We'll be using GPIO 24 but this can be changed later.

- D/C connects to our SPI Chip Select pin. We'll be using GPIO 25, but this can be changed later as well.

1.8" Display

- GND connects to the Raspberry Pi's ground

- Vin connects to the Raspberry Pi's 3V pin

- RST connects to our Reset pin. We'll be using GPIO 24 but this can be changed later.

- D/C connects to our SPI Chip Select pin. We'll be using GPIO 25, but this can be changed later as well.

- CS connects to our SPI Chip Select pin. We'll be using CE0

- MOSI connects to SPI MOSI. On the Raspberry Pi, thats also MOSI

- CLK connects to SPI clock. On the Raspberry Pi, thats SLCK

- LITE connects to the Raspberry Pi's 3V pin. This can be used to separately control the backlight.

SSD1351-based Displays

1.27" and 1.5" OLED Displays

- GND connects to the Raspberry Pi's ground

- Vin connects to the Raspberry Pi's 3V pin

- CLK connects to SPI clock. On the Raspberry Pi, thats SLCK

- MOSI connects to SPI MOSI. On the Raspberry Pi, thats also MOSI

- CS connects to our SPI Chip Select pin. We'll be using CE0

- RST connects to our Reset pin. We'll be using GPIO 24 but this can be changed later.

- D/C connects to our SPI Chip Select pin. We'll be using GPIO 25, but this can be changed later as well.

SSD1331-based Display

0.96" OLED Display

- MOSI connects to SPI MOSI. On the Raspberry Pi, thats also MOSI

- CLK connects to SPI clock. On the Raspberry Pi, thats SLCK

- D/C connects to our SPI Chip Select pin. We'll be using GPIO 25, but this can be changed later.

- RST connects to our Reset pin. We'll be using GPIO 24 but this can be changed later as well.

- CS connects to our SPI Chip Select pin. We'll be using CE0

- Vin connects to the Raspberry Pi's 3V pin

- GND connects to the Raspberry Pi's ground

Setup

You'll need to install the Adafruit_Blinka library that provides the CircuitPython support in Python. This may also require enabling SPI on your platform and verifying you are running Python 3. Since each platform is a little different, and Linux changes often, please visit the CircuitPython on Linux guide to get your computer ready!

Text emphasized with a yellow exclamation: If you have previously installed the Kernel Driver with the PiTFT Easy Setup, you will need to remove it first in order to run this example.

Python Installation of RGB Display Library

Once that's done, from your command line run the following command:

- pip3 install adafruit-circuitpython-rgb-display

If your default Python is version 3 you may need to run 'pip' instead. Just make sure you aren't trying to use CircuitPython on Python 2.x, it isn't supported!

If that complains about pip3 not being installed, then run this first to install it:

- sudo apt-get install python3-pip

DejaVu TTF Font

Raspberry Pi usually comes with the DejaVu font already installed, but in case it didn't, you can run the following to install it:

- sudo apt-get install fonts-dejavu

This package was previously calls ttf-dejavu, so if you are running an older version of Raspberry Pi OS, it may be called that.

Pillow Library

We also need PIL, the Python Imaging Library, to allow graphics and using text with custom fonts. There are several system libraries that PIL relies on, so installing via a package manager is the easiest way to bring in everything:

- sudo apt-get install python3-pil

If you installed the PIL through PIP, you may need to install some additional libraries:

- sudo apt-get install libopenjp2-7 libtiff5 libatlas-base-dev

That's it. You should be ready to go.

Page last edited March 08, 2024

Text editor powered by tinymce.

Python Usage

Text emphasized with a yellow exclamation: If you have previously installed the Kernel Driver with the PiTFT Easy Setup, you will need to remove it first in order to run this example.

Now that you have everything setup, we're going to look over three different examples. For the first, we'll take a look at automatically scaling and cropping an image and then centering it on the display.

Turning on the Backlight

On some displays, the backlight is controlled by a separate pin such as the 1.3" TFT Bonnet with Joystick. On such displays, running the below code will likely result in the display remaining black. To turn on the backlight, you will need to add a small snippet of code. If your backlight pin number differs, be sure to change it in the code:

Copy Code

# Turn on the Backlight

backlight = DigitalInOut(board.D26)

backlight.switch_to_output()

backlight.value = True

Displaying an Image

Here's the full code to the example. We will go through it section by section to help you better understand what is going on. Let's start by downloading an image of Blinka. This image has enough border to allow resizing and cropping with a variety of display sizes and rations to still look good.

Make sure you save it as blinka.jpg and place it in the same folder as your script. Here's the code we'll be loading onto the Raspberry Pi. We'll go over the interesting parts.

Download Project Bundle

Copy Code

# SPDX-FileCopyrightText: 2021 ladyada for Adafruit Industries

# SPDX-License-Identifier: MIT

"""

Be sure to check the learn guides for more usage information.

This example is for use on (Linux) computers that are using CPython with

Adafruit Blinka to support CircuitPython libraries. CircuitPython does

not support PIL/pillow (python imaging library)!

Author(s): Melissa LeBlanc-Williams for Adafruit Industries

"""

import board

import digitalio

from PIL import Image, ImageDraw

from adafruit_rgb_display import (

hx8357,

ili9341,

ssd1331,

ssd1351,

st7735,

st7789,

)

# Configuration for CS and DC pins (these are PiTFT defaults):

cs_pin = digitalio.DigitalInOut(board.CE0)

dc_pin = digitalio.DigitalInOut(board.D25)

reset_pin = digitalio.DigitalInOut(board.D24)

# Config for display baudrate (default max is 24mhz):

BAUDRATE = 24000000

# Setup SPI bus using hardware SPI:

spi = board.SPI()

# Create the display:

# disp = st7789.ST7789(spi, rotation=90,                            # 2.0" ST7789

# disp = st7789.ST7789(spi, height=240, y_offset=80, rotation=180,  # 1.3", 1.54" ST7789

# disp = st7789.ST7789(spi, rotation=90, width=135, height=240, x_offset=53, y_offset=40, # 1.14" ST7789

# disp = st7789.ST7789(spi, rotation=90, width=172, height=320, x_offset=34, # 1.47" ST7789

# disp = st7789.ST7789(spi, rotation=270, width=170, height=320, x_offset=35, # 1.9" ST7789

# disp = hx8357.HX8357(spi, rotation=180,                           # 3.5" HX8357

# disp = st7735.ST7735R(spi, rotation=90,                           # 1.8" ST7735R

# disp = st7735.ST7735R(spi, rotation=270, height=128, x_offset=2, y_offset=3,   # 1.44" ST7735R

# disp = st7735.ST7735R(spi, rotation=90, bgr=True, width=80,       # 0.96" MiniTFT Rev A ST7735R

# disp = st7735.ST7735R(spi, rotation=90, invert=True, width=80,    # 0.96" MiniTFT Rev B ST7735R

# x_offset=26, y_offset=1,

# disp = ssd1351.SSD1351(spi, rotation=180,                         # 1.5" SSD1351

# disp = ssd1351.SSD1351(spi, height=96, y_offset=32, rotation=180, # 1.27" SSD1351

# disp = ssd1331.SSD1331(spi, rotation=180,                         # 0.96" SSD1331

disp = ili9341.ILI9341(

spi,

rotation=90,  # 2.2", 2.4", 2.8", 3.2" ILI9341

cs=cs_pin,

dc=dc_pin,

rst=reset_pin,

baudrate=BAUDRATE,

)

# Create blank image for drawing.

# Make sure to create image with mode 'RGB' for full color.

if disp.rotation % 180 == 90:

height = disp.width  # we swap height/width to rotate it to landscape!

width = disp.height

else:

width = disp.width  # we swap height/width to rotate it to landscape!

height = disp.height

image = Image.new("RGB", (width, height))

# Get drawing object to draw on image.

draw = ImageDraw.Draw(image)

# Draw a black filled box to clear the image.

draw.rectangle((0, 0, width, height), outline=0, fill=(0, 0, 0))

disp.image(image)

image = Image.open("blinka.jpg")

# Scale the image to the smaller screen dimension

image_ratio = image.width / image.height

screen_ratio = width / height

if screen_ratio < image_ratio:

scaled_width = image.width * height // image.height

scaled_height = height

else:

scaled_width = width

scaled_height = image.height * width // image.width

image = image.resize((scaled_width, scaled_height), Image.BICUBIC)

# Crop and center the image

x = scaled_width // 2 - width // 2

y = scaled_height // 2 - height // 2

image = image.crop((x, y, x + width, y + height))

# Display image.

disp.image(image)

Show less View on GitHub

So we start with our usual imports including a couple of Pillow modules and the display drivers. That is followed by defining a few pins here. The reason we chose these is because they allow you to use the same code with the PiTFT if you chose to do so.

Download File

Copy Code

import digitalio

import board

from PIL import Image, ImageDraw

import adafruit_rgb_display.ili9341 as ili9341

import adafruit_rgb_display.st7789 as st7789

import adafruit_rgb_display.hx8357 as hx8357

import adafruit_rgb_display.st7735 as st7735

import adafruit_rgb_display.ssd1351 as ssd1351

import adafruit_rgb_display.ssd1331 as ssd1331

# Configuration for CS and DC pins

cs_pin = digitalio.DigitalInOut(board.CE0)

dc_pin = digitalio.DigitalInOut(board.D25)

reset_pin = digitalio.DigitalInOut(board.D24)

Next we'll set the baud rate from the default 24 MHz so that it works on a variety of displays. The exception to this is the SSD1351 driver, which will automatically limit it to 16MHz even if you pass 24MHz. We'll set up out SPI bus and then initialize the display.

We wanted to make these examples work on as many displays as possible with very few changes. The ILI9341 display is selected by default. For other displays, go ahead and comment out these lines:

Copy Code

disp = ili9341.ILI9341(

spi,

rotation=90,  # 2.2", 2.4", 2.8", 3.2" ILI9341

and uncomment the line appropriate for your display and possibly the line below in the case of longer initialization sequences. The displays have a rotation property so that it can be set in just one place.

Download File

Copy Code

#disp = st7789.ST7789(spi, rotation=90,                            # 2.0" ST7789

#disp = st7789.ST7789(spi, height=240, y_offset=80, rotation=180,  # 1.3", 1.54" ST7789

#disp = st7789.ST7789(spi, rotation=90, width=135, height=240, x_offset=53, y_offset=40, # 1.14" ST7789

#disp = hx8357.HX8357(spi, rotation=180,                           # 3.5" HX8357

#disp = st7735.ST7735R(spi, rotation=90,                           # 1.8" ST7735R

#disp = st7735.ST7735R(spi, rotation=270, height=128, x_offset=2, y_offset=3,   # 1.44" ST7735R

#disp = st7735.ST7735R(spi, rotation=90, bgr=True, width=80,       # 0.96" MiniTFT Rev A ST7735R

#disp = st7735.ST7735R(spi, rotation=90, invert=True, width=80,    # 0.96" MiniTFT Rev B ST7735R

#x_offset=26, y_offset=1,#disp = ssd1351.SSD1351(spi, rotation=180,                         # 1.5" SSD1351

#disp = ssd1351.SSD1351(spi, height=96, y_offset=32, rotation=180, # 1.27" SSD1351

#disp = ssd1331.SSD1331(spi, rotation=180,                         # 0.96" SSD1331

disp = ili9341.ILI9341(

spi,

rotation=90,  # 2.2", 2.4", 2.8", 3.2" ILI9341

cs=cs_pin,

dc=dc_pin,

rst=reset_pin,

baudrate=BAUDRATE

)

Next we read the current rotation setting of the display and if it is 90 or 270 degrees, we need to swap the width and height for our calculations, otherwise we just grab the width and height. We will create an image with our dimensions and use that to create a draw object. The draw object will have all of our drawing functions.

Download File

Copy Code

# Create blank image for drawing.

# Make sure to create image with mode 'RGB' for full color.

if disp.rotation % 180 == 90:

height = disp.width   # we swap height/width to rotate it to landscape!

width = disp.height

else:

width = disp.width   # we swap height/width to rotate it to landscape!

height = disp.height

image = Image.new('RGB', (width, height))

# Get drawing object to draw on image.

draw = ImageDraw.Draw(image)

Next we clear whatever is on the screen by drawing a black rectangle. This isn't strictly necessary since it will be overwritten by the image, but it kind of sets the stage.

Copy Code

# Draw a black filled box to clear the image.

draw.rectangle((0, 0, width, height), outline=0, fill=(0, 0, 0))

disp.image(image)

Next we open the Blinka image, which we've named blinka.jpg, which assumes it is in the same directory that you are running the script from. Feel free to change it if it doesn't match your configuration.

Copy Code

image = Image.open("blinka.jpg")

Here's where it starts to get interesting. We want to scale the image so that it matches either the width or height of the display, depending on which is smaller, so that we have some of the image to chop off when we crop it. So we start by calculating the width to height ration of both the display and the image. If the height is the closer of the dimensions, we want to match the image height to the display height and let it be a bit wider than the display. Otherwise, we want to do the opposite.

Once we've figured out how we're going to scale it, we pass in the new dimensions and using a Bicubic rescaling method, we reassign the newly rescaled image back to image. Pillow has quite a few different methods to choose from, but Bicubic does a great job and is reasonably fast.

Download File

Copy Code

# Scale the image to the smaller screen dimension

image_ratio = image.width / image.height

screen_ratio = width / height

if screen_ratio < image_ratio:

scaled_width = image.width * height // image.height

scaled_height = height

else:

scaled_width = width

scaled_height = image.height * width // image.width

image = image.resize((scaled_width, scaled_height), Image.BICUBIC)

Next we want to figure the starting x and y points of the image where we want to begin cropping it so that it ends up centered. We do that by using a standard centering function, which is basically requesting the difference of the center of the display and the center of the image. Just like with scaling, we replace the image variable with the newly cropped image.

Copy Code

# Crop and center the image

x = scaled_width // 2 - width // 2

y = scaled_height // 2 - height // 2

image = image.crop((x, y, x + width, y + height))

Finally, we take our image and display it. At this point, the image should have the exact same dimensions at the display and fill it completely.

Copy Code

disp.image(image)

Drawing Shapes and Text

In the next example, we'll take a look at drawing shapes and text. This is very similar to the displayio example, but it uses Pillow instead. Here's the code for that.

Download Project Bundle

Copy Code

# SPDX-FileCopyrightText: 2021 ladyada for Adafruit Industries

# SPDX-License-Identifier: MIT

"""

This demo will draw a few rectangles onto the screen along with some text

on top of that.

This example is for use on (Linux) computers that are using CPython with

Adafruit Blinka to support CircuitPython libraries. CircuitPython does

not support PIL/pillow (python imaging library)!

Author(s): Melissa LeBlanc-Williams for Adafruit Industries

"""

import board

import digitalio

from PIL import Image, ImageDraw, ImageFont

from adafruit_rgb_display import (

hx8357,

ili9341,

ssd1331,

ssd1351,

st7735,

st7789,

)

# First define some constants to allow easy resizing of shapes.

BORDER = 20

FONTSIZE = 24

# Configuration for CS and DC pins (these are PiTFT defaults):

cs_pin = digitalio.DigitalInOut(board.CE0)

dc_pin = digitalio.DigitalInOut(board.D25)

reset_pin = digitalio.DigitalInOut(board.D24)

# Config for display baudrate (default max is 24mhz):

BAUDRATE = 24000000

# Setup SPI bus using hardware SPI:

spi = board.SPI()

# Create the display:

# disp = st7789.ST7789(spi, rotation=90,                            # 2.0" ST7789

# disp = st7789.ST7789(spi, height=240, y_offset=80, rotation=180,  # 1.3", 1.54" ST7789

# disp = st7789.ST7789(spi, rotation=90, width=135, height=240, x_offset=53, y_offset=40, # 1.14" ST7789

# disp = st7789.ST7789(spi, rotation=90, width=172, height=320, x_offset=34, # 1.47" ST7789

# disp = st7789.ST7789(spi, rotation=270, width=170, height=320, x_offset=35, # 1.9" ST7789

# disp = hx8357.HX8357(spi, rotation=180,                           # 3.5" HX8357

# disp = st7735.ST7735R(spi, rotation=90,                           # 1.8" ST7735R

# disp = st7735.ST7735R(spi, rotation=270, height=128, x_offset=2, y_offset=3,   # 1.44" ST7735R

# disp = st7735.ST7735R(spi, rotation=90, bgr=True, width=80,       # 0.96" MiniTFT Rev A ST7735R

# disp = st7735.ST7735R(spi, rotation=90, invert=True, width=80,    # 0.96" MiniTFT Rev B ST7735R

# x_offset=26, y_offset=1,

# disp = ssd1351.SSD1351(spi, rotation=180,                         # 1.5" SSD1351

# disp = ssd1351.SSD1351(spi, height=96, y_offset=32, rotation=180, # 1.27" SSD1351

# disp = ssd1331.SSD1331(spi, rotation=180,                         # 0.96" SSD1331

disp = ili9341.ILI9341(

spi,

rotation=90,  # 2.2", 2.4", 2.8", 3.2" ILI9341

cs=cs_pin,

dc=dc_pin,

rst=reset_pin,

baudrate=BAUDRATE,

)

# Create blank image for drawing.

# Make sure to create image with mode 'RGB' for full color.

if disp.rotation % 180 == 90:

height = disp.width  # we swap height/width to rotate it to landscape!

width = disp.height

else:

width = disp.width  # we swap height/width to rotate it to landscape!

height = disp.height

image = Image.new("RGB", (width, height))

# Get drawing object to draw on image.

draw = ImageDraw.Draw(image)

# Draw a green filled box as the background

draw.rectangle((0, 0, width, height), fill=(0, 255, 0))

disp.image(image)

# Draw a smaller inner purple rectangle

draw.rectangle((BORDER, BORDER, width - BORDER - 1, height - BORDER - 1), fill=(170, 0, 136))

# Load a TTF Font

font = ImageFont.truetype("/usr/share/fonts/truetype/dejavu/DejaVuSans.ttf", FONTSIZE)

# Draw Some Text

text = "Hello World!"

(font_width, font_height) = font.getsize(text)

draw.text(

(width // 2 - font_width // 2, height // 2 - font_height // 2),

text,

font=font,

fill=(255, 255, 0),

)

# Display image.

disp.image(image)

Show less View on GitHub

Just like in the last example, we'll do our imports, but this time we're including the ImageFont Pillow module because we'll be drawing some text this time.

Copy Code

import digitalio

import board

from PIL import Image, ImageDraw, ImageFont

import adafruit_rgb_display.ili9341 as ili9341

Next we'll define some parameters that we can tweak for various displays. The BORDER will be the size in pixels of the green border between the edge of the display and the inner purple rectangle. The FONTSIZE will be the size of the font in points so that we can adjust it easily for different displays.

Copy Code

BORDER = 20

FONTSIZE = 24

Next, just like in the previous example, we will set up the display, setup the rotation, and create a draw object. If you have are using a different display than the ILI9341, go ahead and adjust your initializer as explained in the previous example. After that, we will setup the background with a green rectangle that takes up the full screen. To get green, we pass in a tuple that has our Red, Green, and Blue color values in it in that order which can be any integer from 0 to 255.

Copy Code

draw.rectangle((0, 0, width, height), fill=(0, 255, 0))

disp.image(image)

Next we will draw an inner purple rectangle. This is the same color value as our example in displayio quickstart, except the hexadecimal values have been converted to decimal. We use the BORDER parameter to calculate the size and position that we want to draw the rectangle.

Copy Code

draw.rectangle((BORDER, BORDER, width - BORDER - 1, height - BORDER - 1),

fill=(170, 0, 136))

Next we'll load a TTF font. The DejaVuSans.ttf font should come preloaded on your Pi in the location in the code. We also make use of the FONTSIZE parameter that we discussed earlier.

Copy Code

# Load a TTF Font

font = ImageFont.truetype('/usr/share/fonts/truetype/dejavu/DejaVuSans.ttf', FONTSIZE)

Now we draw the text Hello World onto the center of the display. You may recognize the centering calculation was the same one we used to center crop the image in the previous example. In this example though, we get the font size values using the getsize() function of the font object.

Download File

Copy Code

# Draw Some Text

text = "Hello World!"

(font_width, font_height) = font.getsize(text)

draw.text((width//2 - font_width//2, height//2 - font_height//2),

text, font=font, fill=(255, 255, 0))

Finally, just like before, we display the image.

Copy Code

disp.image(image)

Displaying System Information

In this last example we'll take a look at getting the system information and displaying it. This can be very handy for system monitoring. Here's the code for that example:

Download Project Bundle

Copy Code

# SPDX-FileCopyrightText: 2021 ladyada for Adafruit Industries

# SPDX-License-Identifier: MIT

"""

This will show some Linux Statistics on the attached display. Be sure to adjust

to the display you have connected. Be sure to check the learn guides for more

usage information.

This example is for use on (Linux) computers that are using CPython with

Adafruit Blinka to support CircuitPython libraries. CircuitPython does

not support PIL/pillow (python imaging library)!

"""

import subprocess

import time

import board

import digitalio

from PIL import Image, ImageDraw, ImageFont

from adafruit_rgb_display import (

hx8357,

ili9341,

ssd1331,

ssd1351,

st7735,

st7789,

)

# Configuration for CS and DC pins (these are PiTFT defaults):

cs_pin = digitalio.DigitalInOut(board.CE0)

dc_pin = digitalio.DigitalInOut(board.D25)

reset_pin = digitalio.DigitalInOut(board.D24)

# Config for display baudrate (default max is 24mhz):

BAUDRATE = 24000000

# Setup SPI bus using hardware SPI:

spi = board.SPI()

# Create the display:

# disp = st7789.ST7789(spi, rotation=90,                            # 2.0" ST7789

# disp = st7789.ST7789(spi, height=240, y_offset=80, rotation=180,  # 1.3", 1.54" ST7789

# disp = st7789.ST7789(spi, rotation=90, width=135, height=240, x_offset=53, y_offset=40, # 1.14" ST7789

# disp = st7789.ST7789(spi, rotation=90, width=172, height=320, x_offset=34, # 1.47" ST7789

# disp = st7789.ST7789(spi, rotation=270, width=170, height=320, x_offset=35, # 1.9" ST7789

# disp = hx8357.HX8357(spi, rotation=180,                           # 3.5" HX8357

# disp = st7735.ST7735R(spi, rotation=90,                           # 1.8" ST7735R

# disp = st7735.ST7735R(spi, rotation=270, height=128, x_offset=2, y_offset=3,   # 1.44" ST7735R

# disp = st7735.ST7735R(spi, rotation=90, bgr=True, width=80,       # 0.96" MiniTFT Rev A ST7735R

# disp = st7735.ST7735R(spi, rotation=90, invert=True, width=80,    # 0.96" MiniTFT Rev B ST7735R

# x_offset=26, y_offset=1,

# disp = ssd1351.SSD1351(spi, rotation=180,                         # 1.5" SSD1351

# disp = ssd1351.SSD1351(spi, height=96, y_offset=32, rotation=180, # 1.27" SSD1351

# disp = ssd1331.SSD1331(spi, rotation=180,                         # 0.96" SSD1331

disp = ili9341.ILI9341(

spi,

rotation=90,  # 2.2", 2.4", 2.8", 3.2" ILI9341

cs=cs_pin,

dc=dc_pin,

rst=reset_pin,

baudrate=BAUDRATE,

)

# Create blank image for drawing.

# Make sure to create image with mode 'RGB' for full color.

if disp.rotation % 180 == 90:

height = disp.width  # we swap height/width to rotate it to landscape!

width = disp.height

else:

width = disp.width  # we swap height/width to rotate it to landscape!

height = disp.height

image = Image.new("RGB", (width, height))

# Get drawing object to draw on image.

draw = ImageDraw.Draw(image)

# Draw a black filled box to clear the image.

draw.rectangle((0, 0, width, height), outline=0, fill=(0, 0, 0))

disp.image(image)

# First define some constants to allow easy positioning of text.

padding = -2

x = 0

# Load a TTF font.  Make sure the .ttf font file is in the

# same directory as the python script!

# Some other nice fonts to try: http://www.dafont.com/bitmap.php

font = ImageFont.truetype("/usr/share/fonts/truetype/dejavu/DejaVuSans.ttf", 24)

while True:

# Draw a black filled box to clear the image.

draw.rectangle((0, 0, width, height), outline=0, fill=0)

# Shell scripts for system monitoring from here:

# https://unix.stackexchange.com/questions/119126/command-to-display-memory-usage-disk-usage-and-cpu-load

cmd = "hostname -I | cut -d' ' -f1"

IP = "IP: " + subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = "top -bn1 | grep load | awk '{printf \"CPU Load: %.2f\", $(NF-2)}'"

CPU = subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = "free -m | awk 'NR==2{printf \"Mem: %s/%s MB  %.2f%%\", $3,$2,$3*100/$2 }'"

MemUsage = subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = 'df -h | awk \'$NF=="/"{printf "Disk: %d/%d GB  %s", $3,$2,$5}\''

Disk = subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = "cat /sys/class/thermal/thermal_zone0/temp |  awk '{printf \"CPU Temp: %.1f C\", $(NF-0) / 1000}'"

Temp = subprocess.check_output(cmd, shell=True).decode("utf-8")

# Write four lines of text.

y = padding

draw.text((x, y), IP, font=font, fill="#FFFFFF")

y += font.getsize(IP)[1]

draw.text((x, y), CPU, font=font, fill="#FFFF00")

y += font.getsize(CPU)[1]

draw.text((x, y), MemUsage, font=font, fill="#00FF00")

y += font.getsize(MemUsage)[1]

draw.text((x, y), Disk, font=font, fill="#0000FF")

y += font.getsize(Disk)[1]

draw.text((x, y), Temp, font=font, fill="#FF00FF")

# Display image.

disp.image(image)

time.sleep(0.1)

Show less View on GitHub

Just like the last example, we'll start by importing everything we imported, but we're adding two more imports. The first one is time so that we can add a small delay and the other is subprocess so we can gather some system information.

Download File

Copy Code

import time

import subprocess

import digitalio

import board

from PIL import Image, ImageDraw, ImageFont

import adafruit_rgb_display.ili9341 as ili9341

Next, just like in the first two examples, we will set up the display, setup the rotation, and create a draw object. If you have are using a different display than the ILI9341, go ahead and adjust your initializer as explained in the previous example.

Just like in the first example, we're going to draw a black rectangle to fill up the screen. After that, we're going to set up a couple of constants to help with positioning text. The first is the padding and that will be the Y-position of the top-most text and the other is x which is the X-Position and represents the left side of the text.

Copy Code

# First define some constants to allow easy positioning of text.

padding = -2

x = 0

Next, we load a font just like in the second example.

Copy Code

font = ImageFont.truetype('/usr/share/fonts/truetype/dejavu/DejaVuSans.ttf', 24)

Now we get to the main loop and by using while True:, it will loop until Control+C is pressed on the keyboard. The first item inside here, we clear the screen, but notice that instead of giving it a tuple like before, we can just pass 0 and it will draw black.

Copy Code

draw.rectangle((0, 0, width, height), outline=0, fill=0)

Next, we run a few scripts using the subprocess function that get called to the Operating System to get information. The in each command is passed through awk in order to be formatted better for the display. By having the OS do the work, we don't have to. These little scripts came from https://unix.stackexchange.com/questions/119126/command-to-display-memory-usage-disk-usage-and-cpu-load

Download File

Copy Code

cmd = "hostname -I | cut -d\' \' -f1"

IP = "IP: "+subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = "top -bn1 | grep load | awk '{printf \"CPU Load: %.2f\", $(NF-2)}'"

CPU = subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = "free -m | awk 'NR==2{printf \"Mem: %s/%s MB  %.2f%%\", $3,$2,$3*100/$2 }'"

MemUsage = subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = "df -h | awk '$NF==\"/\"{printf \"Disk: %d/%d GB  %s\", $3,$2,$5}'"

Disk = subprocess.check_output(cmd, shell=True).decode("utf-8")

cmd = "cat /sys/class/thermal/thermal_zone0/temp |  awk \'{printf \"CPU Temp: %.1f C\", $(NF-0) / 1000}\'" # pylint: disable=line-too-long

Temp = subprocess.check_output(cmd, shell=True).decode("utf-8")

Now we display the information for the user. Here we use yet another way to pass color information. We can pass it as a color string using the pound symbol, just like we would with HTML. With each line, we take the height of the line using getsize() and move the pointer down by that much.

Download File

Copy Code

y = padding

draw.text((x, y), IP, font=font, fill="#FFFFFF")

y += font.getsize(IP)[1]

draw.text((x, y), CPU, font=font, fill="#FFFF00")

y += font.getsize(CPU)[1]

draw.text((x, y), MemUsage, font=font, fill="#00FF00")

y += font.getsize(MemUsage)[1]

draw.text((x, y), Disk, font=font, fill="#0000FF")

y += font.getsize(Disk)[1]

draw.text((x, y), Temp, font=font, fill="#FF00FF")

Finally, we write all the information out to the display using disp.image(). Since we are looping, we tell Python to sleep for 0.1 seconds so that the CPU never gets too busy.

Copy Code

disp.image(image)

time.sleep(.1)

Page last edited March 08, 2024

Text editor powered by tinymce.

1.8" TFT Shield V2

Let's take a tour of the 1.8" TFT Shield

TFT Display

In the center is the 1.8" TFT display. This display is full color (16-bit RGB), 128x160 pixels, and has a backlight. The display receives data over SPI plus two pins:

- SCK - SPI Clock

- MOSI - SPI Data

- Digital 10 - Chip Select

- Digital 8 - Data/Command Select

The TFT reset is connected to the seesaw chip. The backlight is also PWM controlled by the seesaw chip. The 4 SPI+control pins, however, must be controlled directly by the Arduino

Buttons & Joystick

In addition of the display, you also get a bunch of user-interface buttons.

In the top left is the Reset button, this will reset the shield and Arduino when pressed. It is connected directly to the Reset pins

There are three buttons labeled A B C below the TFT, these are connected to the seesaw chip. You can read the values over I2C

To the right of the TFT is a 5-way joystick. It can be pushed up/down/left/right and select (in). It is connected to the seesaw chip, you can read the joystick over I2C

SD Card

The micro SD card slot can be used to read/write data from any micro SD card using the Arduino libraries. The SD card is connected to the SPI pins as well as Digital #4 for Chip Select

The SD card is not required for use, but it's handy for storing images

seesaw I2C Expander

Instead of taking up a bunch of GPIO pins to read the buttons and joystick, as well as controlling the TFT backlight, we use an I2C expander chip called the seesaw. It is connected to the SDA/SCL pins and can read/write pins with our library. This saves a ton of pins and then you can always use the I2C pins for other sensors, as long as the address doesnt conflict

Don't forget! Since the seesaw chip is used for the TFT backlight and reset, you need to activate it even if you are not reading the buttons or joystick.

Page last edited March 08, 2024

Text editor powered by tinymce.

Testing the Shield

You can test your assembled shield using the example code from the library.

Start by installing a bunch of libraries!

Open the Arduino Library manager

Install the Adafruit GFX Library

If using an older version of the Arduino IDE (pre-1.8.10), also locate and install the Adafruit_BusIO library (newer versions do this automatically when using the Arduino Library Manager).

Adafruit ST7735 Library

Adafruit seesaw Library

You can read more about installing libraries in our tutorial.

Restart the Arduino IDE.

1.8" Shield with seesaw

If your shield looks like this, you have the 1.8" seesaw version (the most recent) which will work with just about any/all boards. For this version load up the seesaw_shield18_test example

Upload to your microcontroller, and open the serial port watcher at 9600 baud:

The sketch waits until the serial port is opened (you can make it auto-start once you know things are working by removing the while (!Serial); line

Check that the seesaw chip is detected, you should see text display on the TFT after a quick draw test.

If you don't have an SD card inserted, it will fail to init the SD card, that's ok you can continue with the test

Once you've gotten this far try pressing all the buttons on the board (except for RESET) to activate the invert-blinking loop.

Text emphasized with a blue exclamation: The graphics don't look identical to the below but you should still press all the buttons as shown!

Your browser does not support the video tag.

For more details about seesaw, check out our guide - we made a nice wrapper for the 1.8" TFT to control the backlight and read buttons but it still might be useful to know the underlying protocol

Displaying a Bitmap

If you have parrot.bmp stored on the SD card you will get a nice parrot display once the buttons have all been pressed

Page last edited March 08, 2024

Text editor powered by tinymce.

CircuitPython Displayio Quickstart

You will need a Metro capable of running CircuitPython such as the Metro M0 Express or the Metro M4 Express. We recommend the Metro M4 Express because it's much faster and works better for driving a display. The steps should be about the same for the Metro M0 Express. If you haven't already, be sure to check out our Adafruit Metro M4 Express featuring ATSAMD51 guide.

Featured Adafruit Metro M4 feat. Microchip ATSAMD51

Are you ready? Really ready? Cause here comes the fastest, most powerful Metro ever. The Adafruit Metro M4...

$27.50 each

In Stock

You could use a Grand Central which also has an M4 Processor. For this board, be sure to check out our Introducing the Adafruit Grand Central M4 Express guide.

Featured Adafruit Grand Central M4 Express featuring the SAMD51

Are you ready? Really ready? Cause here comes the Adafruit Grand Central featuring the Microchip...

$39.95 each

In Stock

If you need WiFi capabilities for your project, you could also use the Metro M4 Airlift Lite. For this board, be sure to check out our Adafruit Metro M4 Express AirLift guide.

Featured Adafruit Metro M4 Express AirLift (WiFi) - Lite

Give your next project a lift with AirLift - our witty name for the ESP32 co-processor that graces this Metro M4....

$34.95 each

In Stock

Preparing the Shield

Before using the TFT Shield, you will need to solder the headers on. Be sure to check out the Adafruit Guide To Excellent Soldering. After that the shield should be ready to go.

Required CircuitPython Libraries

To use this display with displayio, there are a few required libraries. You will need the display driver and since this is no ordinary display and has some additional controls, you will also need the seesaw and busdevice libraries.

First, make sure you are running the latest version of Adafruit CircuitPython for your board.

Next, you'll need to install the necessary libraries to use the hardware--carefully follow the steps to find and install these libraries from Adafruit's CircuitPython library bundle.  Our introduction guide has a great page on how to install the library bundle for both express and non-express boards.

Remember for non-express boards, you'll need to manually install the necessary libraries from the bundle:

- adafruit_st7735r

- adafruit_seesaw

- adafruit_bus_device

Before continuing make sure your board's lib folder or root filesystem has the adafruit_st7735r, adafruit_seesaw and adafruit_bus_device files and folders copied over.

CircuitPython Code Example

Download Project Bundle

Copy Code

# SPDX-FileCopyrightText: 2021 ladyada for Adafruit Industries

# SPDX-License-Identifier: MIT

"""

This example will test out the display on the 1.8" TFT Shield

"""

import time

import board

import displayio

from adafruit_seesaw.tftshield18 import TFTShield18

from fourwire import FourWire

from adafruit_st7735r import ST7735R

# Release any resources currently in use for the displays

displayio.release_displays()

ss = TFTShield18()

spi = board.SPI()

tft_cs = board.D10

tft_dc = board.D8

display_bus = FourWire(spi, command=tft_dc, chip_select=tft_cs)

ss.tft_reset()

display = ST7735R(display_bus, width=160, height=128, rotation=90, bgr=True)

ss.set_backlight(True)

while True:

buttons = ss.buttons

if buttons.right:

print("Button RIGHT!")

if buttons.down:

print("Button DOWN!")

if buttons.left:

print("Button LEFT!")

if buttons.up:

print("Button UP!")

if buttons.select:

print("Button SELECT!")

if buttons.a:

print("Button A!")

if buttons.b:

print("Button B!")

if buttons.c:

print("Button C!")

time.sleep(0.001)

Show less View on GitHub

Let's take a look at the sections of code one by one. We start by importing time, so we can pause, the board so that we can initialize SPI, displayio, the tftshield18 seesaw library, and the adafruit_ili9341 driver.

Download File

Copy Code

import time

import board

import displayio

import fourwire

from adafruit_seesaw.tftshield18 import TFTShield18

from adafruit_st7735r import ST7735R

Next we release any previously used displays. This is important because if the Metro is reset, the display pins are not automatically released and this makes them available for use again.

Copy Code

displayio.release_displays()

We set up seesaw using the TFTShield18, which was written specifically for this shield to make things very easy.

Copy Code

ss = TFTShield18()

Next, we set the SPI object to the board's SPI with the easy shortcut function board.SPI(). By using this function, it finds the SPI module and initializes using the default SPI parameters. Next we set the Chip Select and Data/Command pins that will be used.

Copy Code

spi = board.SPI()

tft_cs = board.D10

tft_dc = board.D8

In the next line, we set the display bus to an instance of FourWire which makes use of the SPI bus.

Copy Code

display_bus = fourwire.FourWire(spi, command=tft_dc, chip_select=tft_cs)

Finally, we reset the display, initialize the driver with a width of 160 and a height of 128, and turn on the backlight. If we stopped at this point and ran the code, we would have a terminal that we could type at and have the screen update.

Copy Code

ss.tft_reset()

display = ST7735R(display_bus, width=160, height=128, rotation=90, bgr=True)

ss.set_backlight(True)

Finally, we place an infinite loop at the end and constantly read the buttons. If a button is detected as being pressed, a message specifies which one. Multiple buttons can be pressed at the same time. We also provide an optional small delay to allow you to adjust how quickly you want the buttons to read in case you want to debounce the output.

Download File

Copy Code

while True:

buttons = ss.buttons

if buttons.right:

print("Button RIGHT!")

if buttons.down:

print("Button DOWN!")

if buttons.left:

print("Button LEFT!")

if buttons.up:

print("Button UP!")

if buttons.select:

print("Button SELECT!")

if buttons.a:

print("Button A!")

if buttons.b:

print("Button B!")

if buttons.c:

print("Button C!")

time.sleep(.001)

Show less

Now go ahead and run the code. Once it's running, try pushing a few buttons and see what happens.

Where to go from here

Be sure to check out this excellent guide to CircuitPython Display Support Using displayio

Page last edited March 08, 2024

Text editor powered by tinymce.

Original V1 Shield

Original V1.0 Shield

If your shield looks like this, you have the original 1.8" TFT shield which does not have a helper seesaw chip

The shield uses the "Classic Arduino" SPI wiring and will perform best with Atmega 328-based Arduinos such as the Uno. It can work with other Arduinos but not very well.

Load up the shieldtest demo

If you are using an Arduino UNO, Duemilanove or compatible with the ATmega328 chipset, you don't have to do anything! If you're using a Mega, Leonardo, Due or other non-ATmega328 chipset, you'll have to make a modification

To use with the shield, modify the example code pin definitions as follows.

Find these lines:

Download File

Copy Code

// Option 1 (recommended): must use the hardware SPI pins

// (for UNO thats sclk = 13 and sid = 11) and pin 10 must be

// an output. This is much faster - also required if you want

// to use the microSD card (see the image drawing example)

Adafruit_ST7735 tft = Adafruit_ST7735(TFT_CS,  TFT_DC, TFT_RST);

// Option 2: use any pins but a little slower!

#define TFT_SCLK 13   // set these to be whatever pins you like!

#define TFT_MOSI 11   // set these to be whatever pins you like!

//Adafruit_ST7735 tft = Adafruit_ST7735(TFT_CS, TFT_DC, TFT_MOSI, TFT_SCLK, TFT_RST);

Text emphasized with a yellow exclamation: This is only required for the V1 shield, the V2 shield uses the hardware SPI port so it's not necessary to use software SPI and in fact it won't work!

The Example code has 2 options for defining the display object. Uno, Duemilanove and other Atmega 328-based processors can use the "Option 1" version of the constructor for best performance:

Copy Code

Adafruit_ST7735 tft = Adafruit_ST7735(TFT_CS,  TFT_DC, TFT_RST);

Mega and Leonardo users should use the "Option 2" version of the constructor for compatibility:

Copy Code

Adafruit_ST7735 tft = Adafruit_ST7735(TFT_CS, TFT_DC, TFT_MOSI, TFT_SCLK, TFT_RST);

Be sure to select only one option and comment out the other with a pair of //'s.

Now upload the sketch to see the graphical display!

Page last edited March 08, 2024

Text editor powered by tinymce.

Assembling the Shield

The shield comes with all surface mount parts pre-soldered. All that remains is to install the headers!

Cut the Header Sections

Cut the breakaway header strip into sections to fit the holes on the edge of the shield. You will need 2 sections of 6-pins and 2 sections of 8 pins.

You can use wire-cutters as shown or pliers to snap them apart between pins.

Insert the Headers into an Arduino

To align the header strips for soldering, insert them (long pins down) into the headers of an Arduino. 

Note that for R3 and later Arduinos, there will be an extra 2 unused pins on the end closest the USB and DC power jacks.

Add the Shield

Place the shield over the header strips so that the short pins stick up through the holes.

And Solder!

Solder each pin to assure good electrical contact. 

For tips on soldering see the Adafruit Guide to Excellent Soldering.

Page last edited March 08, 2024

Text editor powered by tinymce.

Reading the Joystick

The 5-way joystick on the shield is great for implementing menu navigation or even for use as a tiny game controller. To minimize the number of pins required, the joystick uses a different resistor on each leg of the control to create a variable voltage divider that can be monitored with a single analog pin. Each movement of the joystick control connects a different resistor and results in a different voltage reading.

In the code example below, the CheckJoystick() function reads the analog pin and compares the result with 5 different ranges to determine which (if any) direction the stick has been moved. If you upload this to your Arduino and open the Serial Monitor, you will see the current joystick state printed to the screen.

You can use this code as the input method for your menu system or game:

Download File

Copy Code

void setup()

{

// initialize serial communication at 9600 bits per second:

Serial.begin(9600);

}

#define Neutral 0

#define Press 1

#define Up 2

#define Down 3

#define Right 4

#define Left 5

// Check the joystick position

int CheckJoystick()

{

int joystickState = analogRead(3);

if (joystickState < 50) return Left;

if (joystickState < 150) return Down;

if (joystickState < 250) return Press;

if (joystickState < 500) return Right;

if (joystickState < 650) return Up;

return Neutral;

}

void loop()

{

int joy = CheckJoystick();

switch (joy)

{

case Left:

Serial.println("Left");

break;

case Right:

Serial.println("Right");

break;

case Up:

Serial.println("Up");

break;

case Down:

Serial.println("Down");

break;

case Press:

Serial.println("Press");

break;

}

}

Show less

Page last edited March 08, 2024

Text editor powered by tinymce.

Graphics Library

We've written a full graphics library specifically for this display which will get you up and running quickly. The code is written in C/C++ for Arduino but is easy to port to any microcontroller by rewritting the low level pin access functions.

The TFT LCD library is based off of the Adafruit GFX graphics core library. GFX has many ready to go functions that should help you start out with your project. It's not exhaustive and we'll try to update it if we find a really useful function. Right now it supports pixels, lines, rectangles, circles, round-rects, triangles and printing text as well as rotation.

Two libraries need to be downloaded and installed: first is the ST7735 library (this contains the low-level code specific to this device), and second is the Adafruit GFX Library (which handles graphics operations common to many displays we carry). You can install these with the Arduino library manager.

Open up the Arduino library manager:

Search for the Adafruit ST7735 library and install it

Search for the Adafruit GFX library and install it

Check out the GFX tutorial for detailed information about what is supported and how to use it!

Page last edited March 08, 2024

Text editor powered by tinymce.

Troubleshooting

Display does not work on initial power but does work after a reset.

Page last edited March 08, 2024

Text editor powered by tinymce.

Downloads

Files & Datasheets

- Adafruit GFX library

- Adafruit ST7735 library (See our detailed tutorial for installation assistance)

- Fritzing object in the Adafruit library

- Fritzing object for v2 of the 1.8" Shield

- Datasheet for the display

- Datasheet for the display driver chip.

- EagleCAD PCB files for TFT shield

- EagleCAD PCB files for TFT breakout

Breakout Schematic

For the level shifter we use the CD74HC4050 which has a typical propagation delay of ~10ns

Breakout Fabrication print

Shield v2 Schematic & Fab Print

This is the newer seesaw version

Shield V1 Schematic & Fab Print

This is the 'original' non-seesaw version

Page last edited March 08, 2024

Text editor powered by tinymce.