To integrate a GT-U7 GPS module with an ESP32 mini board (such as an ESP32-C3, ESP32-S3 mini, or standard ESP32), connect the UART serial lines and power pins, then use a library like TinyGPS++ in the Arduino IDE to read the NMEA data streams. [1, 2]

Wiring Connections

Connect the GT-U7 module to any available HardwareSerial (UART) pair on your ESP32 mini board. Avoid SoftwareSerial if possible, as hardware serial is much more stable on the ESP32. [1, 2]

| GT-U7 Pin | ESP32 Mini Pin (Example: GPIO 16/17 or default UART2) | Description |
| --- | --- | --- |
| VCC | 3.3V or 5V (Check your specific GT-U7 board specs; many accept 3.3V–5V, though 3.3V logic matches the ESP32) | Power supply |
| GND | GND | Common ground |
| TX | GPIO RX (e.g., GPIO 16) | GPS data output to ESP32 input |
| RX | GPIO TX (e.g., GPIO 17) | ESP32 output to GPS data input (optional if only reading) |

Note: Cross-wire the TX and RX lines—the TX of the GPS goes to the RX pin of the ESP32, and the RX of the GPS goes to the TX pin of the ESP32.

Arduino IDE Code Example

Use the HardwareSerial class built into the ESP32 along with the TinyGPSPlus library to parse the incoming location data. [1, 2]

cpp

#include <TinyGPSPlus.h>

TinyGPSPlus gps;

// Use Serial1 on the ESP32 (adjust RX and TX pins to match your wiring)

#const int RXPin = 16;

#const int TXPin = 17;

void setup() {

Serial.begin(115200);

// Initialize Serial1 with 9600 baud (default for GT-U7)

Serial1.begin(9600, SERIAL_8N1, RXPin, TXPin);

Serial.println("GT-U7 GPS Test Initialized");

}

void loop() {

// Feed incoming serial data to TinyGPS++

while (Serial1.available() > 0) {

if (gps.encode(Serial1.read())) {

displayInfo();

}

}

// If no data is received from the GPS for 5 seconds, warn the user

if (millis() > 5000 && gps.charsProcessed() < 10) {

Serial.println("No GPS data received: check wiring.");

delay(5000);

}

}

void displayInfo() {

if (gps.location.isValid()) {

Serial.print("Latitude: ");

Serial.println(gps.location.lat(), 6);

Serial.print("Longitude: ");

Serial.println(gps.location.lng(), 6);

Serial.print("Altitude: ");

Serial.println(gps.altitude.meters());

} else {

Serial.println("Location: Not Available (Waiting for satellite fix...)");

}

}

Use code with caution.

Tips for First-Time Setup

- Antenna Placement: Make sure the ceramic or external antenna is firmly attached and placed near a window or outdoors. A cold start can take several minutes to acquire a satellite fix. [1, 2]

- Status LED: Look for the onboard LED on the GT-U7 module; once it starts blinking, it has acquired a position fix. [1, 2]

- Baud Rate: The GT-U7 typically defaults to 9600 baud (8 data bits, no parity, 1 stop bit). If it was previously configured to binary/SiRF mode, you may need to send an initialization string or reset it, though most modules boot directly into standard NMEA sentences. [1, 2]

This tutorial demonstrates how to wire and test a GPS module with an ESP32 board using the Arduino IDE:

Could you let me know:

- Which exact ESP32 mini board model you are using (e.g., ESP32-C3 SuperMini, ESP32-S3 Zero)?

- What you plan to build with the GPS data (e.g., a tracker, a clock, or wardriving tool)?

Meshtastic

Heltec® LoRa 32 Peripherals | Meshtastic

Integration: Heltec ESP32 V3 board with GT-U7 GPS/GNSS module for precise location and Real-Time Clock (RTC) tracking. Power Manag...

Seeed Studio Forum

XIAO ESP32C3 - GPS Module - Seeed Studio Forum

HardwareSerial Solution: A user successfully read GPS data on the XIAO ESP32C3 by utilizing `HardwareSerial` instead of `SoftwareS...

Random Nerd Tutorials

ESP32 with NEO-6M GPS Module (Arduino IDE)

Upload the code to your board. Make sure the antenna is connected and that the module or antenna is placed outside or next to a wi...

Show all

ESP32miniC3

Integrating the GT-U7 GPS module with the ESP32-C3 Mini (frequently known as the ESP32-C3 SuperMini) requires careful pin selection. [1, 2]

The ESP32-C3 is a single-core RISC-V chip that handles serial ports differently from classic ESP32 chips. It possesses two hardware UART blocks: UART0 is internally routed through the USB-C port for flashing and serial monitoring, leaving UART1 completely free for your GPS module. [1, 2, 3]

Wiring Setup

Because the ESP32-C3 chip allows you to assign UART functions to almost any GPIO, choose safe, clean pins that do not conflict with strapping constraints (such as GPIO 2, 8, or 9). GPIO 0 (as RX) and GPIO 1 (as TX) are highly reliable choices. [1]

| GT-U7 GPS Pin | ESP32-C3 Mini Pin | Notes |
| --- | --- | --- |
| VCC | 5V (or 3V3) | The GT-U7 module runs smoothly on the board's 5V pin. |
| GND | GND | Ground loop connection. |
| TX | GPIO 0 | GPS Transmit maps to ESP32-C3 Receive line. |
| RX | GPIO 1 | GPS Receive maps to ESP32-C3 Transmit line (optional). |

Note: Ensure your TX and RX lines are cross-wired—the GPS TX pin flows into the ESP32-C3 RX pin, and vice-versa. [1]

Arduino IDE Code

To implement this on the ESP32-C3 platform, define a specific HardwareSerial instance and configure it inside your setup block. [1]

cpp

#include <TinyGPSPlus.h>

// Instantiate the TinyGPS++ software parser

TinyGPSPlus gps;

// Define your chosen GPIOs for UART1 connection

#define RX_PIN 0

#define TX_PIN 1

// Create hardware serial interface 1

HardwareSerial GPSSerial(1);

void setup() {

// Initialize the standard hardware monitor over USB-C

Serial.begin(115200);

// Initialize UART1 linked to your custom pins at the GT-U7's native 9600 baud rate

GPSSerial.begin(9600, SERIAL_8N1, RX_PIN, TX_PIN);

Serial.println("ESP32-C3 & GT-U7 GPS Initialized");

}

void loop() {

// Pass raw NMEA bytes coming from the GPS stream into TinyGPS++

while (GPSSerial.available() > 0) {

if (gps.encode(GPSSerial.read())) {

displayGPSInfo();

}

}

// System warning if no data lines clear after a 5-second interval

if (millis() > 5000 && gps.charsProcessed() < 10) {

Serial.println("Warning: No GPS stream detected. Re-verify wiring loops.");

delay(5000);

}

}

void displayGPSInfo() {

if (gps.location.isValid()) {

Serial.print("Lat: ");    Serial.println(gps.location.lat(), 6);

Serial.print("Lng: ");    Serial.println(gps.location.lng(), 6);

Serial.print("Alt(m): "); Serial.println(gps.altitude.meters());

Serial.print("Sats: ");   Serial.println(gps.satellites.value());

Serial.println("-----------------------------------");

} else {

Serial.println("Scanning... (Locking onto satellite constellation)");

}

}

Use code with caution.

Troubleshooting Tips for the ESP32-C3 Mini

- The "No Output" USB Monitor bug: If Serial.print() fails to output anything on your computer screen even when plugged into USB-C, navigate to the Tools menu in your Arduino IDE and change "USB CDC On Boot" to Enabled.

- Power Draw Over USB: While acquiring satellite fixes, the GT-U7 module draws higher spikes of current. Ensure you are powering the device from a reliable USB port on your PC or an external powered hub.

What kind of IoT application are you tracking or updating with this position data (e.g., streaming over Wi-Fi, saving to flash memory, or displaying on an OLED)?

Random Nerd Tutorials

Getting Started with the ESP32-C3 Super Mini

Sep 20, 2026 — The ESP32-C3 Super Mini is a compact development board featuring the ESP32-C3 chip with built-in Wi-Fi and Bluetooth. this one is ...

Last Minute Engineers

ESP32-C3 Super Mini Pinout Reference

The ESP32-C3 Super Mini is designed to be incredibly easy to use for prototyping. It features two rows of eight pins along its sid...

Google

ESP32-C3 SuperMini – Compact WiFi & Bluetooth 5.0 Module with USB-C

The ESP32-C3 Super Mini is a compact and powerful development board based on the Espressif ESP32-C3FH4 chip. It combines 2.4 GHz W...

Show all

Do I need to upload any software to the GPS module?

No, you do not need to upload any software to the GT-U7 GPS module.

The GT-U7 comes pre-programmed from the factory. As soon as it receives power, it automatically starts calculating its position and continuously streaming raw GPS data (in a standard format called NMEA sentences) out of its TX pin at a 9600 baud rate.

Your only task is to write code for the ESP32-C3, telling it how to listen to that serial stream and parse the text using a library like TinyGPS++.

When would someone modify the module? (Optional)

While not required, some advanced users temporarily change the module's settings using a computer software tool called u-center by u-blox. They might do this to:

- Increase the update rate from 1Hz (once per second) up to 5Hz or 10Hz.

- Change the baud rate from 9600 to a faster speed like 115200.

- Disable unused satellite systems to save a tiny bit of power.

However, for 99% of projects, the factory default settings are exactly what you need. Just wire it up, power it on, and let your ESP32-C3 do the work!

Would you like help with saving the GPS coordinates to a micro SD card, or are you planning to send the location data over Wi-Fi?

Are there libraries for micropython like TinyGPSPlus.h?

Yes, there are several libraries for MicroPython that do the exact same job as TinyGPS++.h.

The most popular and direct alternative is micropyGPS. It is a lightweight, pure-Python NMEA-0183 sentence parser optimized specifically for MicroPython on memory-constrained microcontrollers like the ESP32. [1, 2]

Another common choice is micropython-gps (adapted from Adafruit's CircuitPython GPS parser). [1, 2]

How to use micropyGPS on the ESP32-C3

To get started, download the single micropyGPS.py file from the GitHub repository and upload it directly into your ESP32-C3's filesystem (using an IDE like Thonny). [1, 2]

Here is an example code structure mimicking the Arduino approach using UART1 on GPIO 0 and GPIO 1:

python

import machine

import time

from micropyGPS import MicropyGPS

# 1. Initialize the GPS module parser object

gps_parser = MicropyGPS()

# 2. Setup hardware UART1 on the ESP32-C3

# Using GPIO 0 for RX (connected to GPS TX) and GPIO 1 for TX (connected to GPS RX)

gps_uart = machine.UART(1, baudrate=9600, tx=1, rx=0)

print("ESP32-C3 & GT-U7 MicroPython GPS Initialized")

while True:

# Check if there is serial data waiting in the buffer

if gps_uart.any():

# Read the available bytes

raw_data = gps_uart.read()

# Feed characters to micropyGPS one by one

for byte in raw_data:

# micropyGPS expects string characters (decode bytes to utf-8)

char = chr(byte)

gps_parser.update(char)

# Check if we have a valid structural fix

if gps_parser.clean_sentences > 0:

if gps_parser.latitude[0] != 0.0: # Check if lat is not 0

print("--- GPS Data Acquired ---")

# micropyGPS returns lat/long as [degrees, decimal_minutes, 'direction']

print(f"Latitude: {gps_parser.latitude_string()}")

print(f"Longitude: {gps_parser.longitude_string()}")

print(f"Altitude: {gps_parser.altitude} meters")

print(f"Satellites in Use: {gps_parser.satellites_in_use}")

print(f"Time (UTC): {gps_parser.timestamp[0]}:{gps_parser.timestamp[1]}:{gps_parser.timestamp[2]}")

else:

print("Waiting for satellite fix...")

else:

print("No NMEA sentences received yet. Check your connections.")

time.sleep(1)

Use code with caution.

Key Differences Between TinyGPS++ and micropyGPS

- Coordinate Output: TinyGPS++ outputs coordinates as a flat floating-point decimal (e.g., 37.7749). By default, micropyGPS stores them in a traditional navigation tuple format: [degrees, decimal_minutes, direction]. However, you can instantly display them as flat strings using the built-in .latitude_string() and .longitude_string() methods. [1, 2]

- Time Management: micropyGPS includes a native mechanism to pass in a local timezone offset directly, meaning it can calculate your local target time right inside the parser rather than doing additional timestamp math in your script. [1]

Would you like help with formatting the tuple output into pure decimal degrees, or do you need assistance with a script to log this incoming data to a local file on the ESP32-C3's flash memory?